Circuit Analysis Lab 6

Submitted By:

Waqas Ali

Reg. no.

2025-CE-99

EXPLAINATION OF CIRCUITS

Construction:

- First, take a breadboard or circuit board and gather all the required components such as resistors, DC power supply (RPS), and ammeters.

- Connect the positive terminal of the DC power supply to the circuit.

- Connect a 330 Ω resistor (R1) in series with the positive terminal of the supply.

- After R1, make a junction point where the current will divide into two branches.

- From this junction, connect a 1 kΩ resistor (R2) vertically downwards to the ground (negative terminal of the power supply).

- From the same junction, continue the circuit towards the right and connect a 220 Ω resistor (R3) in series.

- After R3, connect another 1 kΩ resistor (R4) vertically downwards to the ground.

- Connect the negative terminal of the power supply to the common ground line of the circuit.

- Connect an ammeter in series before the junction to measure the total current I1.

- Connect another ammeter in the branch containing R2 to measure current I2.

- Connect a third ammeter in the branch containing R3 and R4 to measure current I3.

- After completing all connections, carefully check the circuit to ensure there are no loose or wrong connections.

- For practical verification, first set the power supply to 5V and note the readings of currents I1, I2, and I3.

- Then increase the supply voltage to 15V and again record all current values.

- Finally, increase the voltage to 25V and take the readings.

- Compare the values and verify that I1=I2+I3, which confirms Kirchhoff’s Current Law (KCL).

Circuit 2

Construction:

- First, take a breadboard or circuit board and collect all required components such as resistors, voltmeters, connecting wires, and a DC power supply (RPS).

- Connect the positive terminal of the DC power supply to the input of the circuit.

- Connect a 330 Ω resistor (R1) in series with the positive terminal.

- Connect a voltmeter (V1) across R1 to measure the voltage drop across it.

- After R1, create a junction point in the circuit.

- From this junction, connect a 1 kΩ resistor (R2) vertically downwards to the ground (negative terminal).

- Connect a voltmeter (V2) across R2 to measure its voltage drop.

- From the same junction, continue the circuit towards the right and connect a 220 Ω resistor (R3) in series.

- Connect a voltmeter (V3) across R3 to measure the voltage drop across it.

- After R3, connect the wire directly downwards to the ground line to complete the loop.

- Connect the negative terminal of the power supply to the ground (common return path).

- Carefully check all connections to ensure correct wiring and avoid loose connections.

- For practical verification, first set the power supply to 5V and note the values of voltages V1, V2, and V3.

- Then increase the supply voltage to 10V and record all voltage readings again.

- Finally, increase the voltage to 15V and take the final readings.

- Compare the values and verify that the supply voltage satisfies E1=V1+V2(or loop equation), which confirms Kirchhoff’s Voltage Law (KVL).